Memorandum 						DRAFT

To:   	   Boulder County Commissioners

From:	   Gordon McCurry, Ph.D., P.G.

Date:	   September 9, 2022

Subject:  Comments on SU-22-0003 Dowe Flats Mining Extension Request

I have reviewed the request by CEMEX, Inc (CEMEX) to amend the existing Special Use Permit SU-93-14 to extend the operations at the Dowe Flats mine area for an additional 15 years and am providing my comments below. For context, I am a professional hydrogeologist with more than 35 years of experience in the areas of surface and groundwater hydrology, including water rights and mining.

The request to extend operations at Dowe Flats is far more than an amendment to the existing permit for the reasons noted below. As a result, this should be considered a new permit request and should have the detailed studies and monitoring plans that would be part of a new permit.

- The proposed mining will extend to the north and east into areas that have not previously been mined. This includes the northern half of the existing mining area of the Hi-Cal + 2nd Ridge Pit, to the south and east of the existing mining area of the 3rd Ridge Pit, between the 3rd and 4th Ridge Pits, to the east and west of the 4th Ridge Pit, and almost all of the land north of the 4th Ridge Pit. Mining on land that is currently undisturbed could create impacts that should be studied and monitored over time, should the permit be approved.

- Contour maps of the proposed mining show that it will extend more than 100 feet deeper than the present topography in many areas, and about 150 feet deep in the northern portion of the Hi-Cal + 2nd Ridge Pit. This will alter the natural groundwater flow in the area with consequences that are unknown and yet not studied.

- Wetlands exist north of the Dowe Flats property. The deep mining in the northern portion of the Dowe Flats property could adversely affect those wetlands and associated habitat.

- The mining will likely extend to the groundwater table and create lakes, similar to those that currently exist. This will expose groundwater to evaporation and also divert shallow alluvial groundwater that otherwise may have discharged into nearby surface waters. The result of these changes to the pre-existing groundwater flow regime may constitute an out-of-priority depletion of the waters of the state. If so, a permit would be required from the State Engineer and a court-approved augmentation plan or approved substitute supply plan would be needed to provide compensatory water to senior water rights appropriators.

In conclusion, the current materials provided by CEMEX are not adequate to evaluate the potential impacts of expanding mining at Dowe Flats. Accordingly, I urge the County Commissioners to deny CEMEX’s request.